Name and Std ID:________________________________________     Lab Section:_____

Date:_________________

PRELAB:

Q1.  Add the following numbers then write them in decimal:

| Binary numbers to add a3 a2 a1 a0 + b3 b2 b1 b0 | Binary result C0 S3 S2 S1 S0 | Decimal conversion D2 D1 (Z2 Y2 X2 W2) (Z1 Y1 X1 W1) |
| --- | --- | --- |
| 1001 + 0111 | 10000 | 16 |
| 1011 + 1001 |  |  |
| 1110 + 0101 |  |  |
| 0010 + 1110 |  |  |
| 1101 + 1011 |  |  |

Q2.  Consider the five-bit binary result (Co, S3, S2, S1, S0) representation in the table above.  We would like to represent each combination as its equivalent in two decimal digits, each of which can be represented in binary as shown in the following table.  Finish filling the following truth table.

Binary Coded Decimal Converter

C0

S3

S2

S1

S0

W2 X2 Y2 Z2

W1 X1 Y1 Z1

| C0 | S3 | S2 | S1 | S0 | Decimal |  | Z2 | Y2 | X2 | W2 | Z1 | Y1 | X1 | W1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 0 | 0 | 0 | 0 | 1 | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 |
| 0 | 0 | 0 | 1 | 0 | 0 | 2 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 0 |
| 0 | 0 | 0 | 1 | 1 | 0 | 3 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 1 |
| 0 | 0 | 1 | 0 | 0 | 0 | 4 | 0 | 0 | 0 | 0 | 0 | 1 | 0 | 0 |
| 0 | 0 | 1 | 0 | 1 | 0 | 5 | 0 | 0 | 0 | 0 | 0 | 1 | 0 | 1 |
| 0 | 0 | 1 | 1 | 0 | 0 | 6 | 0 | 0 | 0 | 0 | 0 | 1 | 1 | 0 |
| 0 | 0 | 1 | 1 | 1 | 0 | 7 | 0 | 0 | 0 | 0 | 0 | 1 | 1 | 1 |
| 0 | 1 | 0 | 0 | 0 | 0 | 8 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | 0 |
| 0 | 1 | 0 | 0 | 1 | 0 | 9 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | 1 |
| 0 | 1 | 0 | 1 | 0 | 1 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 0 |
| 0 | 1 | 0 | 1 | 1 | 1 | 1 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 1 |
| 0 | 1 | 1 | 0 | 0 | 1 | 2 | 0 | 0 | 0 | 1 | 0 | 0 | 1 | 0 |
| 0 | 1 | 1 | 0 | 1 | 1 | 3 | 0 | 0 | 0 | 1 | 0 | 0 | 1 | 1 |
| 0 | 1 | 1 | 1 | 0 | 1 | 4 | 0 | 0 | 0 | 1 | 0 | 1 | 0 | 0 |
| 0 | 1 | 1 | 1 | 1 | 1 | 5 | 0 | 0 | 0 | 1 | 0 | 1 | 0 | 1 |
| 1 | 0 | 0 | 0 | 0 | 1 | 6 | 0 | 0 | 0 | 1 | 0 | 1 | 1 | 0 |
| 1 | 0 | 0 | 0 | 1 |  |  |  |  |  |  |  |  |  |  |
| 1 | 0 | 0 | 1 | 0 |  |  |  |  |  |  |  |  |  |  |
| 1 | 0 | 0 | 1 | 1 |  |  |  |  |  |  |  |  |  |  |
| 1 | 0 | 1 | 0 | 0 |  |  |  |  |  |  |  |  |  |  |
| 1 | 0 | 1 | 0 | 1 |  |  |  |  |  |  |  |  |  |  |
| 1 | 0 | 1 | 1 | 0 |  |  |  |  |  |  |  |  |  |  |
| 1 | 0 | 1 | 1 | 1 |  |  |  |  |  |  |  |  |  |  |
| 1 | 1 | 0 | 0 | 0 |  |  |  |  |  |  |  |  |  |  |
| 1 | 1 | 0 | 0 | 1 |  |  |  |  |  |  |  |  |  |  |
| 1 | 1 | 0 | 1 | 0 |  |  |  |  |  |  |  |  |  |  |
| 1 | 1 | 0 | 1 | 1 |  |  |  |  |  |  |  |  |  |  |
| 1 | 1 | 1 | 0 | 0 |  |  |  |  |  |  |  |  |  |  |
| 1 | 1 | 1 | 0 | 1 |  |  |  |  |  |  |  |  |  |  |
| 1 | 1 | 1 | 1 | 0 |  |  |  |  |  |  |  |  |  |  |
| 1 | 1 | 1 | 1 | 1 |  |  |  |  |  |  |  |  |  |  |

Q3.  Find the logic expressions for Z2, Y2, X2, W2, Z1, Y1, X1, W1 as function of C0, S3, S2, S1 and S0:

Z2 =

Y2 =

X2 =

W2 =

Z1 =

Y1 =

X1 =

W1 =

Q4. Write the verilog code for the Binary Coded Decimal Converter from Section 3.3 using the assign statement.

Example:

module

input …

output …

assign …

endmodule

TA Initials: _________

LAB:

Hardware demonstrates a good design.  TA Initials: _________