Name and Std ID:________________________________________     Lab Section:_____

Date:_________________

PRELAB:

Q1.  Add the following numbers then write them in decimal:

| Binary numbers to add a3 a2 a1 a0 + b3 b2 b1 b0 | Binary result C0 S3 S2 S1 S0 | Decimal conversion N2 N1 (X3 X2 X1 X0) (X3 X2 X1 X0) |
| --- | --- | --- |
| 1001 + 0111 | 10000 | 16 |
| 1011 + 1001 |  |  |
| 1110 + 0101 |  |  |
| 0010 + 1110 |  |  |
| 1101 + 1011 |  |  |

Q2.  Consider the five-bit binary result (C0, S3, S2, S1, S0) representation in the table above.  We would like to represent each combination as its equivalent in two decimal digits, each of which can be represented in binary as shown in the following table.  Finish filling in the following truth table.

Binary Coded Decimal Converter

N2X3 N2X2 N2X1 N2X0

C0

S3

S2

S1

S0

N1X3 N1X2 N1X1 N1X0

| C0 | S3 | S2 | S1 | S0 | Decimal |  | N2X3 | N2X2 | N2X1 | N2X0 | N1X3 | N1X2 | N1X1 | N1X0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 0 | 0 | 0 | 0 | 1 | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 |
| 0 | 0 | 0 | 1 | 0 | 0 | 2 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 0 |
| 0 | 0 | 0 | 1 | 1 | 0 | 3 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 1 |
| 0 | 0 | 1 | 0 | 0 | 0 | 4 | 0 | 0 | 0 | 0 | 0 | 1 | 0 | 0 |
| 0 | 0 | 1 | 0 | 1 | 0 | 5 | 0 | 0 | 0 | 0 | 0 | 1 | 0 | 1 |
| 0 | 0 | 1 | 1 | 0 | 0 | 6 | 0 | 0 | 0 | 0 | 0 | 1 | 1 | 0 |
| 0 | 0 | 1 | 1 | 1 | 0 | 7 | 0 | 0 | 0 | 0 | 0 | 1 | 1 | 1 |
| 0 | 1 | 0 | 0 | 0 | 0 | 8 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | 0 |
| 0 | 1 | 0 | 0 | 1 | 0 | 9 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | 1 |
| 0 | 1 | 0 | 1 | 0 | 1 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 0 |
| 0 | 1 | 0 | 1 | 1 | 1 | 1 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 1 |
| 0 | 1 | 1 | 0 | 0 | 1 | 2 | 0 | 0 | 0 | 1 | 0 | 0 | 1 | 0 |
| 0 | 1 | 1 | 0 | 1 | 1 | 3 | 0 | 0 | 0 | 1 | 0 | 0 | 1 | 1 |
| 0 | 1 | 1 | 1 | 0 | 1 | 4 | 0 | 0 | 0 | 1 | 0 | 1 | 0 | 0 |
| 0 | 1 | 1 | 1 | 1 | 1 | 5 | 0 | 0 | 0 | 1 | 0 | 1 | 0 | 1 |
| 1 | 0 | 0 | 0 | 0 | 1 | 6 | 0 | 0 | 0 | 1 | 0 | 1 | 1 | 0 |
| 1 | 0 | 0 | 0 | 1 |  |  |  |  |  |  |  |  |  |  |
| 1 | 0 | 0 | 1 | 0 |  |  |  |  |  |  |  |  |  |  |
| 1 | 0 | 0 | 1 | 1 |  |  |  |  |  |  |  |  |  |  |
| 1 | 0 | 1 | 0 | 0 |  |  |  |  |  |  |  |  |  |  |
| 1 | 0 | 1 | 0 | 1 |  |  |  |  |  |  |  |  |  |  |
| 1 | 0 | 1 | 1 | 0 |  |  |  |  |  |  |  |  |  |  |
| 1 | 0 | 1 | 1 | 1 |  |  |  |  |  |  |  |  |  |  |
| 1 | 1 | 0 | 0 | 0 |  |  |  |  |  |  |  |  |  |  |
| 1 | 1 | 0 | 0 | 1 |  |  |  |  |  |  |  |  |  |  |
| 1 | 1 | 0 | 1 | 0 |  |  |  |  |  |  |  |  |  |  |
| 1 | 1 | 0 | 1 | 1 |  |  |  |  |  |  |  |  |  |  |
| 1 | 1 | 1 | 0 | 0 |  |  |  |  |  |  |  |  |  |  |
| 1 | 1 | 1 | 0 | 1 |  |  |  |  |  |  |  |  |  |  |
| 1 | 1 | 1 | 1 | 0 |  |  |  |  |  |  |  |  |  |  |
| 1 | 1 | 1 | 1 | 1 |  |  |  |  |  |  |  |  |  |  |

Q3.  Find the logic expressions for N2X3, N2X2, N2X1, N2X0, N1X3, N1X2, N1X1, and N1X0 as a function of C0, S3, S2, S1 and S0:

N2X3 =

N2X2 =

N2X1 =

N2X0 =

N1X3 =

N1X2 =

N1X1 =

N1X0 =

Q4. Write the verilog code for the Binary Coded Decimal Converter from Section 3.3 using the assign statement.

Example:

module

input …

output …

assign …

endmodule

TA Initials: _________

LAB:

Hardware demonstrates a good design.  TA Initials: _________